MOVEFILT.bas TX16W filter table editor and viewer.

1) Make copies of filter table files onto a work disk.

2) Run program and select file to open from displayed directory.

3) Enter and hit return: C for copy, S for swap, N to clear filter (No filter)

-OR-

enter the desired FREQ to look at. Entering "0" means sweep frequencies.

4) Then enter LEVEL to look at; "0" means sweep levels.

5) If FREQ and LEVEL = 0 then the screen is cleared and the directory of files

is displayed again. If no name is entered, then the current file name is

reused and you go back to step 3).

Copy current filter entry:

Choose either to copy to current file or to different file.

Choose which entry location to move filter entry.

Enter new file name if needed.

WILL CREATE NEW FILTER FILE if none of entered name is found.

Swap filter entry:

Only makes swaps within current filter table.

Choose entry location to swap filters with current location.

No Filter:

Makes current entry have a flat frequency response.

MAKEFILT.bas designs a filter and saves to a TX16W filter file.

In general, hitting return (with no new entry) keeps the prior entry.

The Program uses the Remez exchange.  There are 4 parameters to enter:

Number of bands, frequency of bands, gain of bands and weight of bands.

Number of bands: A low pass filter will have two bands: the passband and the

stopband. A peak filter will have three bands: a stop, pass then stop.

Frequency of bands: There are two entries for each edge. A low pass filter has

one edge and needs two frequencies, the start of the edge and the end of the

edge. A peak filter will have 2 edges and needs four frequencies.

Gain of band:  Each band needs a gain less than or equal to "1". Gain is

linear. "1" = 0 db, ".1" = 10 db.

Weight of band: Tells program how much to sacrifice the smoothness of

other bands in order to achieve the desired gain.  Range: 1 to 50 or so. The

ratio of the band weights is the important thing. 2:2 is the same as 1:1.

AFTER all the data is entered, a graph will show the desired frequency

response. If it looks ok, enter "Y".  If you don't like it, you can go back and

change those parameters you want.

If it looks ok, the program does the Remez and then plots the filter response.

If you like what you see, you can save it to any location in an existing or

new filter table file.

A log ( remez.log ) file is also made which records parameters for future

reference in case you made a fabulous filter.