PICA1024.GIF and PICA800.GIF

These are 1024 x 768 and 800 x 600 scanned photos of my 10 inch

coil firing at 2880 VA. The toriod is 20 x 5 inches, the longest

spark recorded here is 72 inches. The coil is powered by a

parallel bank of 12 kV neon sign transformers. Spark gap system

consists of a small rotary gap in series with two cylinder

static gaps. The primary coil was 8.8 turns of 1/2 inch copper

refrigerator tubing, and the tap was set all of the way out. The

tank circuit capacitance consisted of fourteen homemade rolled

capacitors. The value of each cap is ~.019 microfarad rated at

~10 kV pulse; seven capacitors were placed in parallel for a

value of .133 microfarads, and one bank was placed in series on

each side of the primary. The measured capacitance was .0665

microfarads rated at about 20,000 AC volts pulse.

The ten inch secondary coil is wound on sealed PVC pipe, with

just over 1000 turns of number 21 AWG double Formvar coated

magnet wire. Winding length is 32 inches. The system as

pictured here resonates at about 125 kHz.

Photo is a one second exposure on Kodak ASA 400 film. Longer

exposure times bring out the background detail, but superimpose

the sparks to the point they lose definition.

System was designed, built, and operated by Richard T. Quick II

10028 Manchester Rd., Suite 253, Glendale, MO  63122  USA