PICB1024.GIF and PICB800.GIF

These are a second set of (both 1028 x 768 & 800 x 600) photos

of my 10 inch coil firing at 2880 VA. The toriod is 20 x 5

inches, the longest spark recorded here is 72 inches. The coil

is powered by a parallel bank of 12 kV neon sign transformers.

Spark gap system consists of a small rotary gap in series with

two cylinder static gaps. The primary coil was 8.8 turns of 1/2

inch copper refrigerator tubing, and the tap was set all of the

way out. The tank circuit capacitance consisted of fourteen

homemade rolled capacitors. The value of each cap is ~.019

microfarad rated at ~10 kV pulse; seven capacitors were placed

in parallel for a value of .133 microfarads, and one bank was

placed in series on each side of the primary. The measured

capacitance is .0665 microfarads rated at about 20,000 AC volts

pulse.

The ten inch secondary coil is wound on sealed PVC pipe, with

just over 1000 turns of number 21 AWG double Formvar coated

magnet wire. Winding length is 32 inches. The system as

pictured here resonates at about 125 kHz.

Photo is a one second exposure on Kodak ASA 400 film. Longer

exposure times bring out the background detail, but superimpose

the sparks to the point they lose definition.

System was designed, built, and operated by Richard T. Quick II

10028 Manchester Rd., Suite 253, Glendale, MO  63122  USA