PICE1024.GIF and PICE800.GIF

These are scans (1024x768 & 800x600) of Richard Hull's old coil

"Nemesis" producing 10 - 12 foot arcs across his lab. Nemesis is

now dismantled and has been replaced with more efficient Tesla

Magnifier systems. The secondary coil was wound on a sealed thin

wall PVC flume duct fourteen inches in diameter, with a winding

forty-two inches long of No. 18 AWG double Formvar magnet wire.

The secondary had just over 1000 turns and resonated bare at

about 100 kHz. Loaded with the very large toriod discharger this

system resonated around 50 - 60 kHz.

The tank circuit capacitance was only .09 microfarads. The low

tank circuit frequency required to match this secondary is

ahieved with a large 14 turn saucer shaped primary coil tapped

out at over 12 turns. The power supply is a custom wound pole

pig ballasted with both inductance and resistive elements, with

the actual power delivered to the tank circuit at about 10 kVA.

The spark gap system uses a medium sized rotary spark gap with

a vacuum quenching static gap in series. The toriod discharger

is homemade from commercial ventilation ducting and has pro-

tuberances deliberatly added to disrupt the field and force the

spark discharge towards the camera.

Note the electronic test and diagnostic equipment being brutally

struck by the discharges: Richard Hull is no fool, modern solid

state equipment would be junk after this treatment, Richard has

purchased old tube equipment for pennies on the dollar to outfit

his lab. Only tube equipment could withstand this abuse.