PICH1024.GIF and PICH800.GIF

These are scans (1024x768 & 800x600) taken from a B&W photo of a

Tesla coil designed by Jeff Mullins, and built by master coiler

Gary Legel. The entire coil stands five feet from the floor, in-

cluding the 12 x 3 toriod mounted on top of the secondary. The

secondary coil is wound on a three inch diameter by twelve inch

long polystyrene tube and has 700 turns of No. 26 AWG enamel

covered magnet wire closely spaced. The coil is finished with

six coats of polyurethane sealer to reduce corona loss.

The primary coil is an Archimedes spiral (flat "pancake" coil)

consisting of 9 turns of 3/8-inch wide, .02-inch thick beryllium

copper strip tapped at about 7 turns. The turn to turn spacing

is 3/8-inch. Spacing between the first turn of the primary and

the secondary is 3/4-inch. A single open turn of copper is

spaced around the outer turn of the primary to protect the

primary from direct strikes. This open turn is grounded.

The spark gaps consists of four fixed gaps in series. These gaps

were constructed using tungsten tipped electrodes which can be

purchased new from arc welder supply companies.

The capacitor is four .003 microfarad, 30 KV Hi-K Ceramic, RF

rated Sprague "hockey pucks" in parallel. The measured value is

.012 microfarads capacitance with is supplied by two 9000 volt

60 mA neon sign transformers in parallel (only one transformer

was connected when this photo was taken). The input voltage is

controlled by a 22 ampere, 0-140 volt Powerstat.

This coil was built and operated by Gary Legel, 1306 Sheppard

Drive, Fullerton, CA  92631  USA